Overview of Our Approach

Our objective is to extract the mean temperature signal in the periorbital area of the subject through the

Figure 2. System architecture.

course of the interrogation. Fig. 2 depicts the system architecture that realizes this objective. Specifically,

we need a specialized tissue tracker that registers the subject’s periorbital region despite the presence of motion.

Even with the tracking, the extracted temperature signal will be  marred by noise. While some systemic

noise from the electronics of the thermal imaging system is present, the major source of noise will stem

from imperfections in the tracker. Therefore, a noise suppression algorithm (filter) is necessary to improve

the quality of the signal. Finally, we have a pattern recognition algorithm that compares the overall

outlook of the filtered signal (baseline) versus specific instances of high psycho-physiological value

(transient response). This comparison constitutes the basis of a binary classification rule (‘Deceptive’ or

‘Non-Deceptive’).

Visual Facial Tracking relayed to thermal camera

Typically, tracking accuracy can fluctuate, and as long as it is regained momentarily, the effect on end applications (e.g., surveillance) is minimal. However, in physiological monitoring applications such as ours, even temporary

loss of tracking creates serious problems. It results in spikes in the temperature signal of the tracked tissue,

which are falsely indicative of strong physiological responses. This may mislead the pattern recognition algorithm. (The bold print represent the areas of concern which must be overcome in any program)

Tracking is challenging for the periorbital region, since it is a small tissue area and its projection

is greatly affected by any rotation out of the projected image plane (see Fig. 3).

Even when tracking is maintained, its accuracy typically degrades over time.

Simultaneously with the periorbital region it is suggested that tracking a central facial

region that is rich in contrasting features and more invariant to out of plane rotations than the periorbital

region may be more appropriate.  The quality of tracking for this central region is better on average and can be used to correct the estimate of the periorbital tracker. This leads to the extraction of a superior quality temperature signal (see Fig. 4).

Previous research (tamdem tracking) and various new tracking algorithms suggest that a new method of tracking the region of interest may be more appropriate and result in better performance.  It is suggested that use of a visual camera for the tracking of the region of interest may produce better results.  This visual tracking would then be correlated with the thermal tracking camera to analyze the same area for analysis during the questioning of the individual.   For reference, the previous “tandem tracking” system is set forth for reference.  Many of the principals set forth are similar to the visual tracking method.  Further, it is believe that the use of a visual camera for tracking will allow the system to be fully automatic, i.e. the desired region of interest may be automatically detected without manual input from an operator.

Previous Tandem Tracking System and Procedure for Reference

Figure 4. Example of tandem assisted tracking resulted in a superior temperature extraction. On the graph the green (light) line is the temperature extracted for each frame using the tandem assisted tracking, and the red (dark) line is the temperature extracted for each frame using the normal tracking. Note the highlighted portion of the graph where there is a spike at frame 8934 in the temperature extracted from the normal tracker. This is a result of a tracking failure from the normal tracker because of an abrupt movement by the subject. The tandem assisted tracking was able to correctly track the subject through this motion and therefore extract the correct temperature at this frame. The grey scale thermograph on the right shows both the tandem assisted (green/light), and the normal (red/dark) periorbital trackers

The scheme works as follows:

1. The operator selects two rectangular regions on the initial frame of the thermal clip—the central facial

and periorbital.

2. The two regions are tracked by two independent CONDENSATION trackers. The feedback measurement

for each tracker is based on a template extracted from the initial rectangular regions. In essence, this is a sub-sampling of the rectangular region of interest. We have determined experimentally that a surprisingly small sub-sample of the original area produces the same tracking result as the full area.

3. The relative spatial position of the central and periorbital regions is established in the initial frame.

4. In subsequent frames the CONDENSATION trackers come up with independent position estimates.

The position estimate of the central facial tracker is computed first and is used to influence the periorbital

region tracker. Since the relative position of the periorbital region to the central facial region

has been established in the initial frame, the position probability suggested by the central tracker for

the periorbital region is fused into the CONDENSATION framework for the independent estimate

of the periorbital tracker itself. The combined estimate provides for a robust solution and helps the

periorbital tracker to overcome local extrema (see Fig. 5).

Information Fusion

The concept of using the results from one tracker to boost the performance of another tracker has been

previously introduced in the literature. One formulation of this within the context of the CONDENSATION

algorithm is the concept of information fusion through Importance Sampling introduced in Isard and Blake

(1998b) where the performance of a high level tracker, that is a tracker which parameterizes the target object’s

state in a precise manner, is aided by utilizing information from a low level tracker, that is a tracker which

parameterizes the target object in a general manner. We have modified this approach by utilizing the information

from a robust high-level tracker (the central region tracker) to boost the stability of another high-level

tracker (the periorbital region tracker). The benefit of the tandem relationship is enhanced by tuning

the dynamics model for the central region tracker to be more tolerant to large changes in motion (by increasing

the stochastic component of its dynamics model), and tuning the dynamics model for the periorbital region

tracker to allow for more localization (by decreasing the stochastic component of its dynamics model).

Thus far we have omitted the details of how the central region tracker influences the periorbital region

tracker. Incorporating this influence into our algorithm is a two-step process. First, we need to derive the information about the periorbital region tracker’s posterior density from the central region tracker. Then, we need

to have the periorbital region tracker utilize this information in a meaningful way. For our framework let

us assume that Bn = (b1, b2, . . .,bk )T is the general

Figure 5. Tandem Facial Tracking: The subject in the thermograph (shown here in gray-scale) on the left has had his periorbital region tracked

by both the normal (red/dark) and the tandem assisted (green/light) trackers. The normal periorbital region tracker has become confused because

of a local maximum in the template match response space. However, the periorbital region tracker that is assisted by the central region tracker

in the tandem configuration has maintained the proper track. This is because of the suggestion of the central region tracker, which is shown in

the graph on the right as the vertical green (light) line. The response of the template matching along the blue line with endpoints a and b in the

thermograph (see the graph on the right) has been generated to highlight this point.

tracker at frame n, for parameters b1, b2, . . .,bk , where k can be as large as necessary to describe the tracker for

a given representation. Similarly, let us assume that Cn

represents the central region tracker at frame n. In our representation we model the tracked regions by their center of mass positions and orientations.

The state space S for these is given in Eq. (1).

Where x and y are the coordinates of the center of mass of the tracked region, and θ is the angle of the tracked

region with respect to the x-axis of the image coordinate system. The central region tracker uses its current

state, Cn, coupled with information derived during initialization, C0 and B0, to derive the information about

the possible state of the periorbital region tracker Bn (see Eq. (2)).

Where ˆBn represents the projected current state of the periorbital tracker based on the current state of the

central region tracker. ˆBn is generated from the maximum likelihood sample of the central region tracker.

We use the maximum likelihood sample instead of the k-highest weighted samples in order to exert all the influence

of the tandem tracker in one location. Thus, we explore the parameterization space around ˆBn to the

maximum extent possible by virtue of the stochastic component of the dynamics model.

Once ˆBn has been calculated, it must be incorporated into the CONDENSATION framework for

the periorbital tracker. For this purpose we modify the selection step of the CONDENSATION framework

to automatically pre-select a portion of its samples based on this new knowledge. The information

fusion methodology is depicted in Fig. 6,

Figure 6. Modifications to the CONDENSATION framework. On the left is the traditional implementation of the CONDENSATION framework,

on the right is the modified version of the CONDENSATION framework. Notice that during the selection step for the modified version, the

additional information is used as the basis for producing new samples, which would have not been spawned if we used the traditional

where Z is an image and X is the object parameterization;  p (Xt−1|Zt−1) is the posterior probability from

the previous time step derived from the sample set

samples with corresponding weight π(i ); p(Xt |Zt−1) is the prior for time-step t; p(Zt |Xt ) is the observation

density at time step t; p(Xt |Zt ) is the posterior probability for the current time step derived from the sample

set

We assign a percent weight α to the additional information from the central face region tracker. During

the selection step of the CONDENSATION framework we choose (1−α)% of the samples normally and α%

based on the additional information. All of the samples selected using the additional information are set equal

to its suggested state, then during the prediction step only the stochastic component of the dynamics model

is applied to these samples. We assign α = 20%, which we experimentally justified as large enough (greater

than 5% in Fig. 7) to derive the benefit from the tandem tracker’s influence and small enough to allow the

Figure 7. The periorbital tracker’s mean error over 200 frames of video for different percentage weights given to the tandem tracker’s

influence. The error was calculated as the Euclidean distance between the maximum likelihood sample of the periorbital tracker and the

true location of the target. The experiment was performed using an artificial tracking environment.

periorbital tracker to function as a normal CONDENSATION tracker. In Fig. 7 it is apparent that the periorbital tracker needs to have only a small percentage of its samples derived through the tandem tracker’s influence to

achieve maximum accuracy. The experiment was performed under large motion conditions (between 6–8.4

pixels/frame). Under these circumstances the tandem tracker’s influence provided the best approximation of

the current state for the periorbital tracker.

Thermal Templates

Sequential Monte Carlo methods provide frameworks for propagating conditional densities over time, but

omit the vehicle with which to measure the posterior density. The open ended property of these methods

facilitates their adaptation to a myriad of different applications (Doucet et al., 2001), but necessitates the

selection of an appropriate representation and measurement criteria for the phenomena in question. Tracking

research in the thermal infrared spectrum has utilized several different types of representations, from shape

contours (Eveland et al., 2003) to blobs. We have a different end use for our tracking than these other research

efforts; we use the tracker to extract physiological information.

This poses the extra constraint that we need to be able to track any sub region of the face or other

exposed skin. If we utilized the outline of the object to be tracked as the representation of our tracking model,

measurements could be compromised anytime the outline of the tracked object remained the same while the

inside of the object changed, such as a person rotating his/her head out of plane (see Fig. 8).

Figure 8. The problem with outline tracking. The subject’s head has the same outline (drawn in blue) in both frames a and b, even though the

orientation of the subject’s head has changed. If the center of the outline ellipse (the blue dot) was used to extract a temperature reading from

the subject in both frames a and b, the measurements would be taken from different locations.

We opted to adapt the well-known technique of template matching (Barker and Matthews, 2001) to the

thermal modality. The fundamental idea behind template matching is to sub-sample the object of interest,

the template, from the initial frame of video and then to find the region in subsequent frames that most

closely resembles the template. The underlying assumption when using templates to represent the object

of interest is that the appearance of the object will remain relatively constant throughout the course of the

video. In our framework, we work with a thermal video V(c, t), where c = (x, y)T are pixel coordinate pairs and

t is the time of the frame. Templates have been traditionally used in visible spectrum images, but they can

be easily adapted to the thermal modality by letting each pixel c = (x, y)T represent a temperature instead

of a visible light intensity. The thermal template T(c) is extracted from a sub-region R0 of the initial video

frame V(c, 0), which contains the object of interest.

Figure 9. Creation of the periorbital template from the combination of a uniform grid of points and a variance maximized set of points.

To speed up computation a subset of the pixels within the initial region of interest is used for the template.

We chose to compose the set of points in our template T(c) from the union of two subsets, U(c) and M(c)

(see Fig 9).

The set U(c) is composed of points uniformly distributed over the region R0. The set M(c) is composed

of the coldest and hottest pixels from the region R0, which effectively maximizes their variance. The motivation

for composing T(c) as the combination of U(c) and M(c) is to have both spatial and variance information

encoded within the template. Each of U(c) and M(c) contain half of the total number of points designated

for T(c). The set of allowable warps W(c, t; P), where P =(p1, . . ., pk )T is a state vector of parameters, that

the template can experience depends on the expected motion characteristics of the tracked object. A simple

parameterization, assuming an approximately planar object, is described in Eq. (3).

The warp W(c, t; P) maps the pixel c from the initial reference frame of the template T(c) to a sub-pixel location

in the reference frame V(c, t). The formulation for W(c, t; P) can be further condensed to Eq. (4).

For the given model we chose the parameterization

to allow for translation, scaling, and in-plane rotation. To perform a measurement of the

posterior distribution using the previously described thermal template, a triplet containing the state of the

template (as described in Eq. (1)) is needed. This necessitates the mapping from the CONDENSATION sample

parameters to the warp parameters:

p1 = cos (θ)

p2 = sin (θ)

p3 = x_

p4 = y_                           (5)

Equation set (5) includes the parameters of a CONDENSATION sample; x_ and y_ are the center of mass

coordinates for the tracker and θ is the angle of the tracked region with respect to the x-axis of the image

coordinate system. Each sample can be measured by using its x, y, and rotation information (i.e., its position

on the posterior density) to warp the original template onto the current image and perform a point-wise thermal

differencing with the current image. To find the state vector Pt , that represents the optimal template

match for the current frame we perform an optimization on the discrete set of state vectors generated from

the set of CONDENSATION samples (see Eq. (6)).

Initially, we used the estimated posterior mean to generate the state vector Pt that represents the optimal template

match for the current frame. In that approach the target’s estimated motion over time tended to smooth

Figure 10. A sample of two consecutive frames of the simulated tracking environment. We extract a sub-image of a subject’s face from a real

thermal image and then project it around the scene according to the parameters of the experiment. The same thermal image is flipped and is used

to provide a challenging background for the simulation. A blank background could unfairly bias the tracker to always stay with the facial image.

out. Typically, this is a desirable characteristic because it reduces high frequency noise in the signal, but due

to the CONDENSATION process there is a propagation delay whenever there is an abrupt change in the

target object’s dynamics (e.g., the target was moving left then suddenly moved right). This is because most

of the CONDENSATION samples expect the target to continue on its previous motion dynamic, so there

might only be one or two samples that, by virtue of the stochastic component of the prediction step, happen to

catch the target’s correct state. This could still drive the estimated posterior mean close to the true state of the

target, if the few samples that correctly located were weighted much heavier than the rest of the samples.

To realize this we use the maximum likelihood sample within the tandem tracking framework, where one

tracker (the central facial tracker) is acting as a global search tracker and the second tracker (the periorbital

tracker) is acting as a localizing tracker.

Physiological Measurement and Noise Suppression

For every frame we compute the mean temperature of the 10% hottest pixels from within the periorbital region

of interest. We have found experimentally that this represents the mean temperature on the vasculature in

the inner corners of the eyes (see Fig. 16). This correlates with the portion of the periorbital region that

has the highest blood flow rate (that’s why it is the hottest) and is minimally affected in the imagery by

blinking. Therefore, assuming that tracking is accurate, it provides a temperature signal that is indicative

of blood flow perfusion in the eye musculature. We consider that the periorbital temperature signal, as defined

above, consists of several components:

Figure 16. (a) Facial thermal frame of a subject with the periorbital region of interest superimposed. (b) Blow-up of the selected region of

interest with the 10% hottest pixels marked in pink. (c) The periorbital region of interest with its 10% hottest subset superimposed on the facial

and ophthalmic arterio venous complex. The heat convected by the blood flow in this complex is responsible for the elevated temperature with

respect to the rest of the periorbital region. Supply of additional blood to the eye muscle is realized through this complex. Therefore, monitoring

the conduit in the eye corners is sufficient to detect the ‘fight or flight’ syndrome during stress.

1. A low varying component indicative of the long term trend of blood flow levels, which is of high

information value.

2. A mid frequency component, which is associated with temporary disturbances in blood flow caused

by stress in specific Question and Answer (Q&A) sessions. This is also of high information value.

3. A high frequency component caused primarily by tracker instability and systemic noise.

Our noise suppression effort is focused on eliminating the high frequency component. We define the mean

periorbital temperature signal as S [n] , 1 ≤ n ≤ N, where N is the number of frames in the thermal video

clip. We remove the high frequency component of signal S[n] through a novel application of Fourier decomposition.

Specifically, we use the following algorithmic steps:

Step 1. We apply the Fast Fourier Transform (FFT) on signal S[n] to get the power spectrum:

S (ω) = F (S [n])               (7)

The FFT method was first introduced by Cooley and Tukey (1965). Thereafter, it was used widely in signal

analysis due to its high efficiency in comparison to other methods, such as the solution of linear equations or the

correlation method (Smith, 1999). To apply the FFT on the signal S[n], first we use a low order trigonometric

polynomial as follows (Dupros et al., 1992):

u [n] = S [n] = (α cos(n) + β) ,

With

This ensures that the shift will not affect the stability of the scheme by minimizing the Gibbs phenomenon.

Then, we extend u[n] to a 2N periodic function as follows. First, we apply the symmetry (Press et al.,

1992):

and second the periodic extension:

We apply a classical decimation-in-time (Cooley and Tukey, 1965) 1D base-2 FFT method given in Press

et al. (1992).

Step 2. We apply a low-pass filter on the power spectrum S(ω) to suppress high frequency noise. The filter

is constructed using an exponential function:

Step 3. We move back to the time domain by applying the inverse FFT on the filtered spectrum S_(ω):

followed by re-normalization: ˜s [n] = ˜u [n] + (α cos [n] + β) . (13)

The low-pass filtered signal ¯s [n], 1 ≤ n ≤ N (see Figure 17) is fed to the pattern recognition module.

Additionally, an explicit mechanism within our signal processing framework could be employed to ignore

the temperature measurements whenever a serious tracking failure occurs. Such a failure could be ascertained

by monitoring an appropriately defined fitness function. Because the recovery time of tracking failures

in our present data set was generally very small, we chose not to employ such a method. In the future,

however, it may prove advantageous as a first level denoising step to directly segment the temperature signal

into “correctly tracked” and “incorrectly tracked” sections. Depending on the extent and criticality of the

incorrectly tracked sections, they can either be ignored, or the entire signal declared corrupt and no prediction

is being attempted.

Pattern Recognition

We were interested in deriving a decision scheme, which could separate the Deceptive (D) subjects from

the Non-Deceptive (ND) subjects, based on the noisecleaned temperature signals. Our previous research indicated

that elevated levels of stress are associated with elevated periorbital perfusion and temperatures

However, the degree of temperature elevation depends not only on the intensity of stress but also on the psycho-physiology of the subject. Different subjects react with different intensity to the same stress stimuli. Therefore, an effective decision scheme has to normalize inter-individual variability through an intraindividual measure. We compute the slope Di of the filtered temperature signal ˜s[n] that corresponds to the entire length of the interrogation for each subject i. This represents the trend or baseline response of the subject to the experiment. We also compute the slope di 4 of the portion of the signal that corresponds to the Q&A session with the highest impact factor. We define as the impact factor, the level of perceived psychological stress a question has per unit time. In other words, for a question to have a high impact factor it is not only necessary to be ‘tough,’ but also short and feature an

experimentally confirmable lie. It should also be preceded by a series of questions that slowly build pressure

on a subject up to a culmination point. In our interrogation scheme, question 4 qualifies as such a question.

In the computation of di 4 we consider the signal from the beginning of question 4 to the beginning of the respective

answer. Per the psycho-physiological theory this is the interval of interest in a Q&A session.

Our decision scheme is based on the comparison of slope di 4 with Di for subject i:

In other words, if the physiological change in the critical question is greater than the baseline change, then

the subject is classified as deceptive. This binary classification rule can be relaxed by establishing a third

‘indecision’ class when di 4−Di is very close to 0.